Electrical Lighting Controls Schedule

Revision	Date		Description				Created 	Checked

T1		2		Tender	 Issue

Schedule of Equipment – Building wide multi-tenancy control equipment

| Code | Description | iLight Series | Product Code | Image | Areas |
| --- | --- | --- | --- | --- | --- |
| CTRL1 | Backbone Module | iLight Enterprise | BN2-D |  | Communications module for linking floors or zones |
| CTRL2 | TSE Interface, Schedular Timed Events/Scene recall | iLight Enterprise | TSI-1 |  | Building wide control module |
| LCM1 | Auto Addressing 10 Output Pluggable DALI LCM | iLight Enterprise | LCMD-10 |  | Open plan office/meeting rooms |
| LCM2 | 2 Universe DALI Controller (128 addresses) | iLight Enterprise | SCMD2 |  | Open plan office/meeting rooms |
| LCM3 | 4 Universe DALI Controller (256 addresses) | iLight Enterprise | SCMD4 |  | Open plan office/meeting rooms |
| LCM4 | 4 x 32A Relay Module | iLight Enterprise | SCMR0432 |  | For control of non-dimmable luminaires |
| LCM5 | 4 x 1 amp DALI to Phase Adaptive Dimmer (leading / trailing edge) | iLight Enterprise | SCMA0401 |  | For control of architectural mains driven luminaires |
| LCM6 | 12 x 32A Relay Module | iLight Enterprise | SCMR1232 |  | For control of non-dimmable luminaires |
| LCM7 | 12 Digital Dimmer Broadcast DALI / DSI / 1-10V | iLight Enterprise | SCMH1200 |  | Reception scene setting |
| LCM8 | Heating & cooling Controller | iLight Enterprise | HVAC-1 |  | Integrated into a fan coil controller |
| FD1 | DALI field relay On of/off control | iLight Enterprise | FLTS |  | For control of non-dimmable luminaires via DALI |
| P1 | Ceiling mount PIR/photocell 64mm cut out | iLight Enterprise | PPAD- C-DALI-ADDR |  | Recess mounting |
| P2 | Miniature DALI PIR/Photocell 32mmcut out for integration into a luminaire | iLight Enterprise | PPAD- C-MB-DALI ADDR |  | Recess, Luminaire or Besa integrated |
| P3 | Analogue RJ12 connection, 49mm cut out for the iLight Enterprise LCM | iLight Enterprise | AXCS01SRJ |  | Recess mounting |
| P5 | HB DALI Ceiling mount PIR/photocell, 64mm | iLight Enterprise | PPAD-C-HB-DALI-ADDR |  | High Bay - Atriums & Warehouses |
| CP1 | Metal Control Plate- Configurable button sizes & engraving, | iLight Enterprise | Sencia Range |  | Meeting rooms |

| CP2 | Metal Control Plate- 4 Button Raise up/down | iLight Enterprise | Classic Range |  | Meeting rooms |
| --- | --- | --- | --- | --- | --- |
| CP3 | Plastic DALI Control Plate | iLight Enterprise | CLD Range |  | Meeting rooms |
| TS1 | 8” Multiroom Colour Touchscreen | iLight Enterprise | TSE80 |  | Reception, meeting rooms or central controller |
| TS2 | 5.5” Multiroom Colour Touchscreen | iLight Enterprise | TSE55 |  | Reception, meeting rooms or central controller |
| APP | iLight App Lighting, HVAC, Shading | iLight Enterprise | TSEtouch |  | Remote access/meeting rooms for end user applications |
| TS3 | 4” Colour Touch Screen for HVAC-1 Lighting, HVAC & Shading | iLight Enterprise | TSG40 |  | Meeting rooms |
| UI-1 | DALI Input Unit | iLight Enterprise | DALI-I-U |  | Switch input module, via DALI |
| UI-2 | Mini Universal Interface | iLight Enterprise | UIM |  | Connection of switch contacts to CAN and for room partition switches |
| UI-3 | Universal Interface | iLight Enterprise | UIG-2 |  | For end user room partition dividing walls |
| Luminaire leads | 3,4,5, & 6 core leads | iLight Enterprise / Solo | Various |  |  |
| Luminaire plugs | Standard & emergency | iLight Enterprise / Solo | Standard ACCPL-6P-01-B Emergency ACCPL-6P-01-R |  |  |